PRO ENGINEERING PLATFORM

Scope Verification & Elemental Cost Estimation Process

Full Process After Client Job Request · AECOM-Method Adapted for Urban Residential Turnkey

| Stage | Title | Actor | Timing | Output |
| --- | --- | --- | --- | --- |
| 1 | Job request received — NO estimate issued | Platform (automated) | Immediate | Acknowledgement to client. Visit booking triggered. |
| 2 | Authorised Lead assigned & visit booked | Platform coordinator + AI console | Within 4 hours | Lead profile sent to client. Visit time confirmed. |
| 3 | Site visit: 12-point scope checklist completed | Lead Engineer (on site) | Within 24 hrs of request | Completed checklist, 8+ photos, measurements, risk flags — all on platform app. |
| 4 | Scope report submitted to Pro Engineering | Lead Engineer (via platform app) | Within 2 hrs of visit | Verified scope report with risk flags and measured quantities. |
| 5 | Elemental cost estimate generated | Platform estimate engine (automatic) | Within 30 min of report acceptance | Elemental breakdown: scope × zone rate card × benchmark data. |
| 6 | Verified scope report + estimate posted to client | Platform (automated) + coordinator review | Within 24 hrs of visit | Client sees: photos + scope + risk explanations + elemental quote + zone benchmark. |
| 7 | Client accepts, queries, or requests value engineering | Client (response on platform) | Client's own pace | Accepted quote → 1% token payment. Query → Lead contacts client. VE request → revised scope options shown. |

Core principle (AECOM-method): no estimate is issued before scope is verified. The client's job description is treated as a brief, not as a scope. The Lead's site visit converts the brief into a verified scope from which the estimate is built.

Stage 1 — Job Request Received: No Estimate Issued

Actor: Platform (automated system). Timing: Immediate on submission.

What the platform does:

- Records the client's job request: job type, zone, requested start date, service model (R1 Turnkey).

- Runs a pre-qualification check: (a) Is the job type within platform scope? (b) Is the zone active with a minimum viable worker pool? (c) Is the requested start date ≥72 hours ahead?

- Does NOT generate an estimate. Does NOT show a yardstick figure. The previous method of showing a cost range at this stage is removed entirely.

- Sends an acknowledgement message to client (see text below).

- Triggers the visit booking process (Stage 2).

Acknowledgement message sent to client (verbatim):

"Thank you for your job request. A Pro Engineering Authorised Lead will contact you within 4 hours to schedule a brief site visit (30–45 minutes at your home/site). There is no charge for the site visit. Your verified scope report and itemised cost estimate will be shared within 24 hours of the visit. No work will be priced or committed before the site visit is complete."

Why this matters (AECOM principle):

AECOM's founding rule: 'Garbage in, garbage out.' A client who says 'I want to renovate my bathroom' has given a brief, not a scope. Pricing a brief produces an estimate that will always be wrong — either overpriced (and client walks away) or underpriced (and platform loses margin). The site visit converts the brief into a scope. Only then is an estimate valid.

Stage 2 — Authorised Lead Assigned & Visit Booked

Actor: Platform coordinator + AI availability console. Timing: Within 4 hours of job request.

Lead assignment criteria (AI console checks all simultaneously):

| Criterion | Minimum requirement | Why |
| --- | --- | --- |
| Zone | Lead must be registered in the client's service zone | No cross-zone deployment for visit — local knowledge is essential for accurate scope |
| Rating | Deployment rating ≥4.0/5.0 | Below this threshold: scope errors and client complaints are statistically more frequent |
| Relevant experience | Minimum 5 completed jobs of the same scope type | A Lead who has never supervised a bathroom renovation cannot reliably scope one |
| Availability | Available on date client requests for visit and for project start | Prevents booking a Lead who then cannot take the project |
| Certification | Site Safety Supervisor Certificate active | Mandatory for all Leads on any residential site |

What is sent to the client when Lead is assigned:

- Lead's full name and photograph.

- Platform rating (e.g., 4.7 / 5.0) with the number of completed similar jobs (e.g., '52 bathroom renovations completed').

- A direct contact number for confirming the visit time.

- Two visit slot options: Morning (9:00–12:00) or Afternoon (2:00–5:00). Client selects one.

- Confirmation that the visit is free of charge and non-committal.

What the Lead receives before the visit (via platform app):

- Client name, address, and contact number.

- Job type and requested start date.

- 12-point scope checklist pre-loaded (ready to complete on site — see Stage 3).

- Zone rate card: current labour and material rates for this zone and scope type.

- Past job photos from similar scopes completed in the zone — visual reference for condition assessment.

- Platform rule: NO price is to be discussed with the client during the visit.

Stage 3 — Site Visit: 12-Point Scope Verification Checklist

Actor: Lead Engineer (on site at client's home). Timing: Within 24 hours of job request. Duration: 30–45 minutes on site.

The 12-point scope checklist (completed on platform app, not paper):

| # | Scope check point | What the Lead does / records | Output / what is uploaded |
| --- | --- | --- | --- |
| 1 | Bathroom dimensions — actual measured | Lead measures actual length × width × height with tape measure. Records in sq ft (floor area) and sq ft (wall tile area separately). Never uses client's stated figure. | Measured sq ft (floor), measured sq ft (wall tile area including cut tiles + 10% waste factor), ceiling height. Photograph of measuring. |
| 2 | Wall condition | Lead inspects all four walls: tile adhesion (tap test), damp patches, cracks (structural vs. shrinkage vs. settlement), surface preparation requirement. | Condition rating: Good / Fair / Poor per wall. Photographs of each wall. Risk flag if structural cracks found. |
| 3 | Floor condition | Existing tile condition, sub-floor integrity, slope to drain (check with spirit level), drain trap type and condition. | Condition rating. Floor fall measurement. Drain trap type noted. Photograph. |
| 4 | Waterproofing — existing condition | Evidence of existing waterproofing failure: damp stains on external walls or ceiling below, efflorescence on tiles, tile lifting, leakage complaints from client. | Risk flag raised if evidence of existing waterproofing failure. Photographs of evidence. Risk cost estimate pre-populated in checklist. |
| 5 | Plumbing — visible condition | Location and condition of visible supply pipes (copper/CPVC/uPVC), isolation valve locations, drain fall adequacy, existing fixture conditions. | Pipe material and visible condition noted. Risk flag if aged copper or galvanised pipe. Photograph of pipe runs. |
| 6 | Electrical — panel and circuits | Existing electrical panel location, circuits serving the bathroom (switch board, geyser point, exhaust fan, lights), earthing adequacy, wet-zone fitting compliance (IP44 minimum). | Existing fittings IP rating noted. Risk flag if non-compliant. Electrician scope confirmed: geyser point, exhaust, lights. |
| 7 | Fixtures — scope to replace | Comprehensive list: EWC (make, model, floor/wall-hung), wash basin (type), shower mixer (concealed/exposed), geyser (capacity, existing location), accessories (towel rail, soap dish, paper holder). | Complete fixture list. Client preference noted (brand, colour). Photograph of each existing fixture. |
| 8 | Tile specification | Tile type: ceramic / vitrified / large format (600×1200) / natural stone. Grade. Pattern (straight / diagonal / herringbone). Feature wall required? Client's own tiles or platform to supply? | Tile spec confirmed. If client supplies own tiles: Lead notes brand, confirms Lead time before start date. If platform supplies: zone vendor confirmed. |
| 9 | Ceiling scope | Material: RCC (paint only) / false ceiling (gypsum board to install / replace). Height. Exhaust duct direction. Wet zone boundary (which portion is wet zone requiring IP fittings). | Ceiling scope confirmed. Gypsum area if applicable. Exhaust duct direction noted. Photograph. |
| 10 | Site access and logistics | Building access: lift or staircase. Rubble disposal: common area rules, nearest disposal point or society dumpster location. Working hours permitted by building society. | Access route photographed. Rubble disposal plan noted. Working hours restriction noted (affects duration calculation). |
| 11 | Structural check | If any wall is proposed for removal or modification: Lead checks if it is structural (loadbearing) or non-structural. Any structural check must be flagged for engineer review — Lead does not approve structural demolition. | Risk flag mandatory if any wall modification is in scope. Platform will require structural engineer confirmation before approval. |
| 12 | 8+ site photographs uploaded | Minimum 8 photographs: each wall, floor, ceiling, every fixture, plumbing access, electrical panel, external wall adjacent to bathroom (to check for moisture). | Photos auto-tagged with checklist point reference on platform app. Upload completes site visit record. |

Lead conduct rules during the visit:

- NO price is discussed with the client at any point during or after the visit.

- The Lead says: 'I will submit my report to Pro Engineering. You will receive the scope report and estimate within 24 hours.'

- If client asks for a ballpark: 'I am not in a position to give you a number — the estimate is generated by the platform from the verified scope. It will be more accurate than anything I say now.'

- The Lead is a technical observer and recorder — not a salesperson on site.

- All checklist items must be completed and uploaded before leaving the site. Partial submissions are not accepted.

Stage 4 — Scope Report Submitted to Pro Engineering

Actor: Lead Engineer (via platform app, immediately after visit). Timing: Within 2 hours of site visit. Review by: Platform coordinator.

Scope report auto-generated by platform from checklist data:

- Measured quantities: floor area (sq ft), wall tile area (sq ft including waste factor), ceiling area (sq ft).

- Scope of work: itemised list of all work confirmed by checklist (demolition, waterproofing, tiling, plumbing, electrical, painting).

- Risk flags: each flag listed with Lead's description, photograph references, and the platform's standard risk cost allowance for that flag type.

- Duration: platform calculates working days based on scope complexity — not assumed to be the client's stated duration.

- Lead notes: free-text observations uploaded by Lead on any non-standard condition.

- Photographs: all 8+ photographs embedded in the report, tagged to the checklist point they document.

Platform coordinator review (mandatory before estimate is generated):

| Review check | Pass criteria | Action if fail |
| --- | --- | --- |
| Checklist completeness | All 12 points completed. No blank fields. | Return to Lead for completion. Estimate not triggered. |
| Photograph count | Minimum 8 photographs. Each tagged to checklist point. | Return to Lead. Visit record incomplete. |
| Measurements present | Floor area, tile area, and ceiling height all captured in numbers — not descriptions. | Return to Lead for re-measurement. |
| Risk flags reviewed | Each risk flag reviewed by coordinator. Standard cost allowance confirmed as appropriate. | Override if non-standard risk — coordinator sets custom allowance. |
| Duration plausibility | Platform-calculated duration checked against coordinator's experience with similar jobs. | Override if obviously incorrect (e.g., checklist suggests very complex scope but system calculated 2 days). |
| Scope completeness | All items in the client's original request are accounted for in the checklist. | Add missing item with coordinator note — return to Lead for confirmation if significant. |

If any check fails: report is returned to the Lead with specific correction required. Estimate is not triggered until the report passes all six checks. This is the quality gate — it is the platform's equivalent of AECOM's 'scope verification before pricing' rule.

Stage 5 — Elemental Cost Estimate Generated

Actor: Platform estimate engine (automatic). Timing: Within 30 minutes of report acceptance. Basis: AECOM-method elemental cost plan adapted for residential turnkey.

The estimation method — how the platform calculates the price:

- Parametric estimation using internal benchmark data: the platform applies a zone-calibrated rate card to each measured quantity from the verified scope. This is the AECOM parametric method — cost per unit of output × verified quantity.

- Zone rate card: rates are set at actual market rates (not statutory minimums) for the zone, updated quarterly from: (a) platform's own completed job cost data, (b) PWD/CPWD published Schedule of Rates, (c) zone material price surveys.

- Elemental structure: the estimate is broken into elements (demolition, waterproofing, tiling, plumbing, electrical, painting, supervision, compliance, platform fee, contingency). Each element is priced separately from the verified scope.

- Risk item pricing: each risk flag from the checklist is priced as a named line item. It is not absorbed into a round-number contingency — it is itemised so the client can see exactly what they are paying for and can choose to accept or waive.

- Duration drives labour cost: working days (platform-calculated from scope complexity) × daily labour rates. NOT a flat yardstick — actual scope-driven duration.

Zone rate card — Pro Engineering (Delhi South zone, bathroom renovation, Jun 2025):

| Element | Unit | Zone rate | Basis | Source |
| --- | --- | --- | --- | --- |
| Demolition & rubble removal | ₹/sq ft | ₹52–60 | Per sq ft of area demolished + rubble hauled | Platform completed jobs + CPWD 2024 |
| Waterproofing — standard (crystalline) | ₹/sq ft | ₹125–140 | Per sq ft of treated area | Material cost (Pidilite/SikaHermetik) + application labour |
| Waterproofing — risk layer (additional) | Lump sum | ₹2,500–3,200 | Per instance of existing leak at floor-wall junction | Platform rework cost database + material cost |
| Floor tiling — vitrified Grade 1 (600×600) | ₹/sq ft | ₹175–200 | Supply + fix including adhesive and grout | Zone tile vendor rates + platform labour rate card |
| Floor tiling — ceramic (300×300 or 300×600) | ₹/sq ft | ₹135–155 | Supply + fix | Zone vendor + labour |
| Wall tiling — ceramic (300×600) | ₹/sq ft | ₹150–175 | Supply + fix including adhesive and grout | Zone vendor + labour |
| Plumbing — EWC (supply + installation) | Lump sum | ₹3,500–4,500 | Standard wall-hung or floor-mounted EWC, including connection | Platform completed job data |
| Plumbing — wash basin (supply + fix) | Lump sum | ₹1,800–2,500 | Counter / wall-mounted, including mixer tap if supplied | Platform completed job data |
| Plumbing — shower mixer (concealed) | Lump sum | ₹2,500–3,500 | Including concealed body, flush trim, hand shower | Platform data |
| Plumbing — geyser connection point | Lump sum | ₹1,200–1,800 | Supply and pressure test | Platform data |
| Plumbing — pipe replacement (copper, per running metre) | ₹/running metre | ₹350–480 | CPVC replacement including fittings | Material + labour |
| Electrical — exhaust fan (supply + install + duct) | Lump sum | ₹1,800–2,400 | Including IP54 ceiling fan, duct run up to 1.5m | Platform data |
| Electrical — downlight (supply + wire + IP44 fitting) | Per light | ₹900–1,200 | LED, IP44 rated for wet zone | Platform data |
| Electrical — geyser power point (15A) | Lump sum | ₹800–1,100 | Including MCB, weatherproof socket | Platform data |
| Ceiling emulsion (PU putty + 2 coats) | ₹/sq ft | ₹42–50 | Including PU putty, sanding, 2 coats emulsion | Zone painter rate card |
| Supervision — Lead Engineer | ₹/day | ₹1,100–1,300 | Full day on site | Platform Tier-1 rate card |
| Statutory compliance | % of labour | 16% | PF 12% + ESI 1.5% + workmen's insurance 2.5% | Statutory rates |
| Platform service fee | % of labour + compliance | 22% | Governance, matching, quality, technology | Platform margin target |
| GST | % of total | 18% | On labour + compliance + service fee | GST Act on labour supply |
| Contingency | Flat % | 5% | Named risk reserve — returned to client if unused | Standard professional practice (T&T/AECOM benchmark) |

Example elemental estimate — verified scope, bathroom renovation, Delhi South (64 sq ft, 6 days, standard specification + 2 risk items):

| Element | Verified scope / qty | Rate applied | Amount (₹) |
| --- | --- | --- | --- |
| Demolition & rubble removal | 64 sq ft full strip | ₹55/sq ft | ₹ 3,520 |
| Waterproofing — standard | 64 sq ft floor + 2 ft wall perimeter | ₹130/sq ft | ₹ 8,320 |
| Waterproofing — risk layer ⚠ | Additional layer: existing leak at floor-wall junction (risk flag) | Lump sum | ₹ 2,800 |
| Floor tiling | 64 sq ft vitrified 600×600 Grade 1 | ₹185/sq ft (supply+fix) | ₹ 11,840 |
| Wall tiling | 13 sq ft feature wall, ceramic 300×600 | ₹160/sq ft (supply+fix) | ₹ 2,080 |
| Plumbing — EWC replacement | New wall-hung EWC, complete installation | Lump sum | ₹ 4,200 |
| Plumbing — wash basin | New counter-top basin, mixer tap | Lump sum | ₹ 2,400 |
| Plumbing — shower mixer (concealed) | Concealed body + flush trim + hand shower | Lump sum | ₹ 3,200 |
| Plumbing — geyser connection | 15A supply point + pressure test | Lump sum | ₹ 1,400 |
| Plumbing — pipe replacement ⚠ | Aged copper supply line: full replacement (risk flag) | Lump sum | ₹ 3,500 |
| Electrical — exhaust fan | IP54 fan, duct run 1.2m | Lump sum | ₹ 2,100 |
| Electrical — downlights | 2 × IP44 LED downlights, wired | 2 × ₹1,050 | ₹ 2,100 |
| Electrical — geyser point | 15A MCB + weatherproof socket | Lump sum | ₹ 950 |
| Ceiling emulsion | 64 sq ft PU putty + 2 coats emulsion | ₹45/sq ft | ₹ 2,880 |
| Supervision — Lead Engineer | 6 days × ₹1,100/day | ₹1,100/day | ₹ 6,600 |
| Statutory compliance | PF + ESI + workmen's insurance | 16% of labour cost | ₹ 3,648 |
| Platform service fee | Governance + quality + matching + technology | 22% of (labour + compliance) | ₹ 5,100 |
| GST | 18% on labour + compliance + service fee | 18% | ₹ 4,092 |
| Contingency — named risk reserve | 5% of subtotal (returned if unused) | 5% | ₹ 2,500 |
| TOTAL — Scope-verified estimate (±8% accuracy) | All elements above | AECOM-method elemental | ₹ 72,180 |

Zone benchmark comparison (shown to client alongside the estimate): Average cost for bathroom renovation of this specification in Delhi South in the past 90 days: ₹61,500–₹74,200. This estimate (₹72,180) falls within the zone benchmark range. The client can verify this is a market-correct price, not a platform-specific invention.

Accuracy class disclosure (shown to client): This is a Scope-Verified Estimate. Accuracy: ±8%. Basis: Measured quantities from site visit + zone rate card from 47 completed platform jobs in Delhi South + PWD/CPWD rates. This is more accurate than a description-based estimate (±25%) but less detailed than a full Bill of Quantities (±3%). For a standard bathroom renovation of this type, ±8% is the professional standard accuracy class.

Stage 6 — Verified Scope Report & Estimate Posted to Client

Actor: Platform (automated delivery) + coordinator sign-off. Timing: Within 24 hours of site visit. Delivery: Platform app notification + link to report page.

What the client receives on the report page (all in one view):

| Section | What the client sees | Purpose |
| --- | --- | --- |
| 1. Scope photographs | 8+ photographs of the existing bathroom, tagged to the scope elements they document | Client sees the condition we found. No surprises — they were there. |
| 2. Verified scope summary | Plain-language summary: 'We found: full demolition required, existing floor-wall leak, aged copper pipe, all fixtures to replace…' | Client confirms we understood the job correctly. |
| 3. Measurement corrections (if any) | If measured area differs from client's stated area: 'Your bathroom is 64 sq ft, not 60 sq ft. The additional 4 sq ft adds ₹X to tile cost. Here is the measured drawing.' | Transparent explanation of any difference. No surprise cost increases after the job starts. |
| 4. Risk flag explanations (plain language) | Each risk item explained in non-technical language. E.g.: 'We found an existing leak at the floor-wall junction. If this is not treated before tiling, your new tiles will lift within 12–18 months and the bathroom will leak again. We have included the treatment in the estimate (₹2,800). If you prefer to proceed without it, you can waive this item — see below.' | Educates the client. Gives them a real choice. Removes the fear of 'they added extra costs.' |
| 5. Elemental cost breakdown | Full table: every element, scope, rate basis, and amount. Total = ₹72,180. | Transparency — client sees where every rupee goes. |
| 6. Zone benchmark comparison | 'Average for this specification in Delhi South (past 90 days): ₹61,500–₹74,200. Your estimate: ₹72,180 — within range.' | Third-party credibility anchor. Client can independently verify we are market-correct. |
| 7. Accuracy class statement | 'This is a Scope-Verified Estimate. Accuracy: ±8%. Basis: measured quantities + zone rate card.' | Honest professional standard. Builds trust. |
| 8. Lead profile + availability confirmation | Supervisor assigned: Rakesh Sharma (4.7/5, 52 bathroom renovations). Available from your start date. | Client knows the person who will do the work before paying anything. |
| 9. Response options | (a) Accept — pay 1% token. (b) Query an item — Lead will call within 2 hours. (c) Request value engineering — show me options within a lower budget. (d) Waive a risk item — sign a waiver acknowledging the risk. | Client is in control. No pressure to accept blindly. |

Stage 7 — Client Response: Accept, Query, Value Engineer, or Waive

Actor: Client (response on platform). The platform responds differently to each choice.

| Client response | What the platform does | Timeline | Outcome |
| --- | --- | --- | --- |
| Accept estimate as-is | Client taps 'Accept'. Platform generates 1% token payment screen. Booking is locked. | Immediate | Token paid → Team assembled → Booking confirmed. |
| Query one or more line items | Client marks the line items they want explained. Platform notifies Lead Rakesh Sharma. Lead calls client within 2 hours to explain. | Lead calls within 2 hrs | Once query resolved: client accepts or re-queries. Platform records the conversation. |
| Request value engineering (budget is lower) | Client states their budget. Platform generates 2–3 scope revision options showing what the budget achieves: • Option A: standard ceramic floor tiles instead of vitrified (saves ₹2,400) • Option B: retain existing EWC, replace seat only (saves ₹3,500) • Option C: single downlight instead of 2 (saves ₹800) Client chooses combinations. Revised estimate auto-updated. | Within 1 hour of request | Revised estimate shown. Client accepts or iterates once more. |
| Accept but waive a risk item | Client taps 'Waive' on a specific risk item. Platform generates a digital waiver: 'I acknowledge that Pro Engineering has recommended [item]. I choose to proceed without this treatment. I accept the consequential risk (tile lifting, pipe failure, etc.) and will not hold Pro Engineering liable for outcomes related to this decision.' Client signs digitally. | Immediate | Revised estimate (minus waived item) shown. Client accepts revised total and pays token. |
| Decline entirely | Client declines. No fee charged. Lead is thanked and released. The scope report and photos are retained in the platform database for future benchmark data. | Immediate | Job cancelled. Zone benchmark data updated from this scope report (even though no work was done — the pricing data is still valid). |

Value engineering principle (Jacobs design-to-cost + WSP value engineering):

When a client's budget is below the platform estimate, the platform does NOT ask the client to reduce team size or duration — the client does not have the technical knowledge to make that decision. Instead, the platform proposes specific scope modifications (material substitutions, fixture upgrades deferred, work phased) and calculates the resulting cost saving for each. The client chooses from defined options. The technical decision remains with the platform; the commercial decision belongs to the client.

Post-acceptance: what happens before work begins:

- Booking token (1%) collected. Amount is credited against first milestone payment — not an additional charge.

- Team assembled from zone pool: crew composition is based on verified scope, not a flat yardstick. If the scope has an aged copper pipe, a plumber with pipe replacement experience is assigned — not the next available plumber.

- Milestone payment schedule issued to client: 30% mobilisation / 40% waterproofing + tiling complete / 30% snag-free handover.

- Quality gates pre-scheduled in the delivery plan: waterproofing ponding test (24-hour), plumbing pressure test before wall close, IP rating check for electrical fittings — all arise from the scope report, not from a generic protocol.

- All risk flag resolutions noted in the delivery plan: if the client accepted the waterproofing risk layer, it is a confirmed line item in the delivery plan. If waived, the waiver document is attached to the delivery record.

- 48-hour cancellation window: full token refund if cancelled within 48 hours of booking confirmation.

Why This Process is the Professional Standard — Firm-by-Firm Benchmark

| Stage | Pro Engineering process element | Benchmark firm it mirrors | What it replaces (old method) |
| --- | --- | --- | --- |
| 1 — No estimate before site visit | Zero estimate on submission. Acknowledgement only. | AECOM: 'No estimate before scope is verified.' T&T: scope drives the cost plan. | Old: auto-generated yardstick estimate shown immediately. Inaccurate and undercuts credibility. |
| 2 — Authorised Lead assignment | AI console assigns zone-registered, rated, experienced Lead. Profile shared with client before visit. | Akkodis: coverage stress-test before bid. Mace: pre-named team at bid stage. | Old: anonymous 'platform team' with no identity shared with client. |
| 3 — 12-point scope checklist | Mandatory, photographed, app-based checklist. All 12 points required before report is submitted. | AECOM: structured scope verification methodology. Jacobs: stage-gate process. | Old: client's verbal description accepted as scope. |
| 4 — Report quality gate | Platform coordinator reviews report against 6 criteria before triggering estimate. | T&T: QA review before cost plan is released to client. Jacobs: stage-gate approval. | Old: no review gate. Estimate auto-generated from whatever description was entered. |
| 5 — Elemental estimate from verified measurements | Every element priced from measured quantities × zone rate card. Not yardstick × minimum wage. | T&T: elemental cost plan (RICS NRM method). AECOM: parametric from benchmark database. | Old: headcount × days × minimum wage. Inaccurate by 15–35% in practice. |
| 6 — Zone benchmark shown to client | Platform's zone average for the same specification shown alongside the estimate. | Hays: Salary Guide as public rate anchor. T&T: global benchmark data as proposal attachment. | Old: ±10% band with no external reference. Client had no basis to trust the range. |
| 7 — Design-to-budget value engineering | If client budget is below estimate: platform proposes named scope options (not headcount cuts) to reach the budget. | Jacobs: design-to-cost. WSP: formal value engineering as scope optimisation. | Old: 'Reduce team size or duration.' Placed technical decision on a non-expert client. |
| 7 — Risk waiver option | Client can waive named risk items by signing a digital waiver acknowledging the specific risk. | Mace: open-book with client choice. AECOM: risk register with explicit risk-acceptance process. | Old: risk items buried in contingency. Client did not know they existed. |

Single most important principle of this entire process: the estimate is a consequence of the scope, not a starting point for the conversation. Every firm in the benchmark (AECOM, T&T, Mace, Jacobs, WSP) arrives at price after scope. The platform's old method arrived at price before scope. That is the fundamental difference between a crude estimate and a professional one.