PRO ENGINEERING PLATFORM

Supervisor Site Visit App — Complete Specification

Screen-by-screen design · All 7 forms integrated · Field logic · Risk flags · Platform rules

This document is the complete specification for the Supervisor Site Visit App. It integrates all 7 forms from the uploaded Mason Site Visit document with the platform's 12-point checklist design. Every screen is described with its exact fields, input types, validation rules, risk flag triggers, and platform logic. This is ready for development.

App Overview — Screen Map

| Screen | Title | Form source | Type | Locked until |
| --- | --- | --- | --- | --- |
| Screen 1 | Job Details View | Platform data | Read only — no input | Nothing — opens automatically on job assignment |
| Screen 2 | Demolition & Structural Condition | Form 1 — Item 1 | Checklist + photograph | Screen 1 viewed |
| Screen 3 | Plumbing & Core Alterations | Form 1 — Item 2 | Checklist + measurements + photograph | Screen 2 submitted |
| Screen 4 | Electrical Condition | New — platform addition | Checklist + photograph | Screen 3 submitted |
| Screen 5 | Waterproofing Assessment | New — platform addition | Checklist + risk flag + photograph | Screen 4 submitted |
| Screen 6 | Wall Condition & Plastering Need | Form 1 — Item 1 (wall substrate) | Checklist + photograph | Screen 5 submitted |
| Screen 7 | Floor Condition & Screed Requirement | Form 1 — Item 1 (floor slab) | Checklist + measurement | Screen 6 submitted |
| Screen 8 | Tile Specification | Form 4 — Material matrix | Selection + quantity calculation | Screen 7 submitted |
| Screen 9 | Fixture & Sanitaryware Scope | New — platform addition | Checklist | Screen 8 submitted |
| Screen 10 | Ceiling Scope | New — platform addition | Checklist + photograph | Screen 9 submitted |
| Screen 11 | Site Access & Vertical Logistics | Form 1 — Item 3 + Form 7 | Checklist + measurement | Screen 10 submitted |
| Screen 12 | Community Compliance | Form 6 | Checklist acknowledgement | Screen 11 submitted |
| Screen 13 | Measurements — Actual Verified | All forms | Numbers entry — all dimensions | Screen 12 submitted |
| Screen 14 | Labour Requirement Sheet | Form 2 | Auto-calculated from measurements | Screen 13 submitted |
| Screen 15 | Material Estimation | Form 4 | Auto-calculated + brand tier selection | Screen 14 submitted |
| Screen 16 | Risk Flag Summary | All screens | Auto-generated from all flags raised | Screen 15 submitted |
| Screen 17 | Timeline — Day-by-Day Schedule | Form 3 | Auto-generated from scope + curing norms | Screen 16 viewed |
| Screen 18 | Payment Milestone Schedule | Form 5 | Auto-calculated from labour cost | Screen 17 viewed |
| Screen 19 | Submit Scope Report | All forms | Final submission | All 18 screens complete — submit button unlocked |

Submit button on Screen 19 is DISABLED until all 18 previous screens are marked complete. The app shows a progress bar: '12 of 18 screens complete.' Each screen turns green when submitted. Supervisor cannot leave the site and submit later from memory — all photographs must be uploaded live on site. Platform coordinator is notified the moment Screen 19 is submitted.

Screen-by-Screen Specification

| 1 | Job Details View Read only. Supervisor taps to start the visit. No input on this screen. → Client name and registered mobile (display only — not editable) → Job address — full address with floor number and building name → Zone: Noida Sector 62 / 63 / 64 (or other) → Job type: Home renovation — Bathroom → Service model: R1 Turnkey / R2 / R3 / R4 / R5 (as booked) → Assigned supervisor: [Supervisor name] — that is you → Scope description from client booking (as entered at Step 1 of booking flow) → Client's stated budget (from Step 2 of booking flow) — shown for reference only → Client's preferred start date → Button: 'Start site visit' — tapping this timestamps the visit start and activates Screen 2 |
| --- | --- |

| 2 | Demolition & Structural Condition Form 1 — Item 1 ¦ Photograph mandatory → Floor slab status — tap one: Tile-over-tile possible (no height restriction) / Full demolition required down to base concrete slab → If full demolition selected: estimated total surface area to demolish (sq ft) — number entry. Norm: 100 sq ft per shift for demolition labour. → Wall substrate condition — tap one: Walls are plumb — direct tile adhesive possible / Walls are uneven — full mortar plastering layer required before tiling → Existing wall tile height — number entry in feet → Target new wall tile height — number entry in feet → Are any walls proposed for removal? — Yes / No → If Yes: is the wall load-bearing? — Yes (RISK FLAG AUTO-RAISED) / No / Not sure (RISK FLAG AUTO-RAISED) → Structural cracks visible? — Yes (RISK FLAG) / No — if Yes: describe location and type in notes field → Notes field — free text for any additional observation → Photograph button — minimum 2 photographs: floor slab condition + wall substrate condition. Upload locked until both taken. |
| --- | --- |

| ⚠ | RISK FLAG auto-raised if: load-bearing wall removal proposed, or structural cracks found. Flag label: 'Structural risk — civil engineer assessment required before work begins.' Cost allowance: to be determined by coordinator. Work on structural scope BLOCKED until coordinator approves. |
| --- | --- |

| 3 | Plumbing & Core Alterations Form 1 — Item 2 ¦ Photograph mandatory → Toilet (WC) relocation required? — No shift required / Relocation required → If relocation: distance of shift — number entry in feet → If relocation: slab intervention needed? — Requires core cutting through slab (RISK FLAG) / Rerouting within floor bed height only → Shower line relocation required? — No shift required / Relocation required → If shower relocation: distance of shift — number entry in feet → If shower relocation: wall chasing (grooving) required in brick wall for hidden lines? — Yes / No → Visible supply pipe material — GI (old, corroded risk) / Copper / CPVC / uPVC / Not visible → If GI or copper pipe visible: estimated age — less than 10 years / 10–20 years / over 20 years (RISK FLAG if over 20 years) → Drain pipe condition — Good / Slow drain observed (RISK FLAG) / Blocked → Notes field → Photograph button — minimum 2: existing plumbing access point + drain location |
| --- | --- |

| ⚠ | RISK FLAG auto-raised if: core cutting through slab required — label 'Core cutting required: adds 1 day, structural approval needed.' OR aged pipe over 20 years — label 'Aged supply pipe: replacement recommended. Cost allowance: ₹3,500.' OR slow drain — label 'Drain blockage: jetting required before work starts. Cost allowance: ₹800.' |
| --- | --- |

| 4 | Electrical Condition Platform addition ¦ Photograph mandatory → Existing exhaust fan — Present and working / Present but not working / Absent → Existing light fittings in wet zone — IP44 or higher (compliant) / Standard fitting not IP rated (RISK FLAG) / Absent → Geyser — Present and working (retain) / Present but not working / Absent (new point required) → Geyser capacity if present — 6L / 10L / 15L / 25L → Switch board location — Inside bathroom / Outside bathroom → Earth leakage protection (ELCB/RCD) on bathroom circuit — Present / Absent (RISK FLAG) / Not known → Number of new electrical points required: exhaust fan point / 2-pin socket point / geyser point / light points — number entry for each → Notes field → Photograph button — minimum 1: existing electrical fittings and switch board |
| --- | --- |

| ⚠ | RISK FLAG auto-raised if: non-IP rated fittings in wet zone — label 'Non-compliant wet zone fittings: replacement to IP44 mandatory. Cost allowance: ₹1,200.' OR no ELCB/RCD — label 'No earth leakage protection: ELCB installation recommended for safety. Cost allowance: ₹800.' |
| --- | --- |

| 5 | Waterproofing Assessment Platform addition — most critical check ¦ Photograph mandatory → Evidence of existing waterproofing failure — damp patch on adjacent wall or ceiling below: Yes (RISK FLAG) / No → If Yes: location of damp patch — floor-wall junction / wall-wall corner / ceiling below / multiple locations → Efflorescence (white salt deposits) visible on tiles or walls — Yes (RISK FLAG) / No → Tile lifting or hollow sound on tap test — Yes (RISK FLAG) / No → Existing waterproofing system visible (if any) — Crystalline visible / Bituminous membrane visible / Not visible / Unknown → Sunken toilet slab — Yes (additional waterproofing scope required) / No → Proposed waterproofing system — 2-coat elastomeric (standard) / Crystalline (upgrade) / Cementitious tanking (heavy duty for multiple failures) → Waterproofing area: floor only / floor + 300mm wall skirt (standard) / floor + full wall height → Notes field — describe exact location of all damp evidence → Photograph button — minimum 3: damp patch (if present), tile hollow area, floor-wall junction condition |
| --- | --- |

| ⚠ | RISK FLAG auto-raised if: any damp evidence found — label 'Existing waterproofing failure: additional treatment layer required. Standard cost allowance: ₹2,800.' If multiple failure locations: 'Severe waterproofing failure: heavy duty tanking system recommended. Coordinator review required before estimate.' |
| --- | --- |

| 6 | Wall Condition & Plastering Requirement Form 1 — Item 1 (wall substrate) ¦ Photograph mandatory → North wall condition — Good (no plastering needed) / Fair (minor patch repair) / Poor (full replastering required) → South wall condition — Good / Fair / Poor → East wall condition — Good / Fair / Poor → West wall condition — Good / Fair / Poor → For each wall rated Poor: is the existing plaster debonded (hollow on tap test)? — Yes / No → Total wall area requiring replastering — auto-calculated from measurements in Screen 13 but supervisor can override → Plastering specification required — Rough plaster base only (for tiling over) / Smooth finish plaster (for painting) / Skim coat only over existing plaster → Notes field → Photograph button — minimum 4: one photograph of each wall showing its condition |
| --- | --- |

| 7 | Floor Condition & Screed Requirement Form 1 — Item 1 (floor slab) ¦ Photograph mandatory → Existing floor condition — Intact tiles, sound / Tiles hollow or debonded / Bare concrete slab → Floor fall to drain — Adequate (water flows to drain freely) / Inadequate (water pools) / Cannot determine (existing tiles in place) → Drain type — P-trap (standard, retain) / Floor trap (retain or replace) / Unknown → Drain condition — Clear and functional / Slow / Blocked → New screed required — Yes, full floor screed with slope / Yes, partial screed to correct fall only / No (tile directly over existing screed) → Screed specification — Standard cement-sand 1:4 / Waterproof screed with additive (Cico/Fosroc) / Self-levelling compound → Floor fall direction — toward centre drain / toward corner drain / toward door threshold drain → Notes field → Photograph: minimum 2 — floor surface condition + drain location and type |
| --- | --- |

| 8 | Tile Specification & Material Selection Form 4 — Material matrix ¦ No photograph required → Floor tile format — 2×2 ft (600×600mm) / 2×4 ft (600×1200mm) / 1×1 ft (300×300mm) / Other (specify) → Floor tile type — Matt vitrified / Glossy vitrified / Matt ceramic / Anti-skid → Floor tile brand tier — Premium (Kajaria/Nitco/Johnson) / Value (Somany/AGL/Cera) / Budget (Orientbell/local brand) → Wall tile format — 2×2 ft / 1×2 ft (300×600mm) / 1×1 ft / Other → Wall tile type — Glossy ceramic / Matt ceramic / Glossy vitrified / Digital print → Wall tile brand tier — Premium / Value / Budget → Skirting tile — 4-inch integrated skirting (same as floor tile) / No skirting / Separate skirting tile → Feature wall tile — Yes (different tile for one wall) / No → Tile adhesive type — Standard cement-based / High-flex polymer-modified (for large format or wet zone) / Rapid-set → Grout type — Standard cement grout / Epoxy grout (premium, stain-proof) — epoxy recommended for wet zone → Client supplies own tiles — Yes (platform measures and orders via coordinator) / No (platform sources) → Quantities auto-calculated from measurements entered in Screen 13 — supervisor confirms or adjusts for waste factor (standard: +10%) |
| --- | --- |

| 9 | Fixture & Sanitaryware Scope Platform addition ¦ No photograph required at this screen (photographs from Screen 3) → EWC (toilet): Replace / Retain / Not in scope → If replace: wall-hung or floor-mounted? / preferred brand tier: Premium (Kohler/American Standard/Jaquar) / Value (Cera/Hindware/Parryware) / Budget → Wash basin: Replace / Retain / Not in scope → If replace: counter-top / pedestal / wall-hung / under-counter → Shower system: Replace with new / Retain existing / Upgrade to concealed mixer → If concealed mixer: exposed pipe + rose only / thermostatic concealed valve + rose → Geyser: New point only / Replace geyser unit / Retain → Towel rail / robe hook / toilet paper holder / mirror — include or not for each → Taps and mixers — Replace all / Replace only damaged / Retain all → Number of accessories to be supplied and fitted — total count for estimate |
| --- | --- |

| 10 | Ceiling Scope Platform addition ¦ Photograph mandatory → Existing ceiling type — RCC slab (painted directly) / False ceiling — gypsum / False ceiling — PVC / False ceiling — wood → Ceiling scope — Paint only (PU putty + 2 coats emulsion) / Replace false ceiling / New false ceiling / No ceiling work → Exhaust fan duct direction — Through east wall / Through west wall / Through external wall / Existing duct retained → Exhaust duct run length — number entry in feet → Ceiling height — number entry in feet (also captured in Screen 13 measurements) → Wet zone boundary — first 600mm from shower head (IP44 lighting mandatory within this zone) → Number of new light points in ceiling — number entry → Notes field → Photograph: minimum 1 — existing ceiling condition and exhaust duct location |
| --- | --- |

| 11 | Site Access & Vertical Logistics Form 1 — Item 3 + Form 7 → Building floor — Ground / 1st / 2nd / 3rd / 4th / 5th / Higher → Lift access — Service lift available / Passenger lift only (rubble and materials via staircase) / No lift (staircase only) → Staircase width — can a loaded sack trolley fit? — Yes / No (manual carrying only) → Distance from staircase exit to material loading/unloading vehicle spot — number entry in metres → Rubble disposal point — society-designated chute / ground floor compound area / skip bin on road / no designated point (coordinator must arrange) → Expected floor height rise after new tile laying — number entry in mm → Door bottom clearance after height rise — Door clears comfortably / Door must be trimmed by carpenter (additional scope + cost) → Bulk material storage — ground floor compound available / inside flat only / landing / no storage space available → Just-in-time lift protocol required? — Yes (materials brought up in batches, maximum 20 boxes at a time) / No → Building society working hours restriction — Standard (09:00–18:00) / Restricted (specify hours in notes) / No restriction → High-noise work hours (heavy chipping, cutting) — platform default: Mon–Fri 09:00–13:00 and 16:00–18:30. Saturday 10:00–13:00 only. Sunday: NO work. → Notes field — any access, parking, or logistics issue not covered above → Photograph: minimum 1 — staircase/lift access and rubble disposal route |
| --- | --- |

| 12 | Community Compliance Acknowledgement Form 6 — mandatory before any work begins → Supervisor acknowledges and confirms the following on behalf of the platform and crew: → High-noise work (chipping, grinding, pneumatic tools) restricted to: Mon–Fri 09:00–13:00 and 16:00–18:30 / Saturday 10:00–13:00 only / Sunday and public holidays: NO WORK → Daily stairway sweep — any sand, cement dust, or debris on public stairs or landing swept clean before 18:00 every day: Acknowledged → Structural protection — heavy tile boxes and cement bags not dragged across granite steps or common area flooring: Acknowledged → Structural load safety — debris bags moved downstairs immediately, not piled on landing, corridor, or balcony: Acknowledged → Common area damage — any damage to lift interior, corridor walls, lobby floor, or staircase caused by the crew is the platform's liability. Supervisor photographs existing damage before Day 1: Acknowledged → Supervisor digital signature — Aadhaar-linked OTP confirmation that all 6 compliance points are acknowledged → Note: if any compliance restriction is more stringent than the platform default (e.g., society permits work only 10:00–12:00), it is entered in the notes field here and auto-applied to the timeline in Screen 17 |
| --- | --- |

| 13 | Measurements — Actual Verified Dimensions All forms — the foundation of all quantity calculations → INSTRUCTION: All measurements taken by supervisor with tape measure on site. Client's stated dimensions are shown for reference but NEVER used for calculations. → Bathroom length — number entry in feet and inches → Bathroom width — number entry in feet and inches → Calculated floor area — auto-calculated: length × width in sq ft (shown immediately on entry) → Ceiling height — number entry in feet and inches → Wall 1 (north) — length in feet and height in feet → Wall 2 (south) — length in feet and height in feet → Wall 3 (east) — length in feet and height in feet → Wall 4 (west) — length in feet and height in feet → Total wall area — auto-calculated from the four walls → Deductions from wall area — door opening width × height, window opening width × height — entered separately, auto-deducted → Net wall tile area — auto-calculated: total wall area minus deductions, plus 10% waste factor → Existing wall tile height (if retaining lower section) — number entry if partial tile replacement → New tile target height — number entry → Shower niche dimensions — width × height × depth (if applicable) → Skirting length — auto-calculated as perimeter minus door width → If client stated area vs measured area differ by more than 5%: ALERT shown — 'Measured area differs from client-stated area. Estimate will use measured figure. Note explained to client in scope report.' → Photograph of measuring tape on each dimension — 4 photographs minimum |
| --- | --- |

| 14 | Labour Requirement Sheet Form 2 — auto-calculated from measurements + scope selections → All quantities are auto-populated from Screen 13 measurements and scope selections in Screens 2–12. → Each row shows: Trade / Industry output norm / Total quantity / Calculated man-days / Supervisor override field → Demolition labour (skilled chipper): norm 100 sq ft per 8-hour shift. Quantity: total demolition area from Screen 2. Calculated man-days shown. Supervisor confirms or overrides. → Debris handling (helper): norm 10 bags per hour. Quantity: estimated bags from demolition area ÷ 7 sq ft per bag. Floor count adjustment: +0.5 man-days per floor above ground. → Plumbing technician (skilled): norm 1 complete bathroom routing per 16 hours. Quantity: 1 if no relocation, 2 if toilet + shower relocated, 3 if core cutting also required. → Plumbing helper: 1 helper per plumber per shift. Man-days = plumber man-days. → Base plastering mason: norm 150 sq ft per 8-hour shift. Quantity: net wall area requiring plastering from Screen 6. → Floor screeding mason: norm 150 sq ft per 8-hour shift. Quantity: floor area from Screen 13. → Civil helper (mortar mixing): 1 helper per mason. Man-days = plastering + screeding mason days. → Tile layout specialist (skilled): norm 60–70 sq ft per 8-hour shift for 2×2 format. Quantity: total tile area (floor + walls). For large format (600×1200): norm reduces to 40–50 sq ft per shift. → Tile helper: 1 per tile mason. Man-days = tile mason days. → Epoxy grout applicator: norm 200 sq ft per 8-hour shift. Quantity: total grouting area. → Waterproofing applicator: norm 100 sq ft per 8-hour shift. Quantity: waterproofing area from Screen 5. → Electrician (Wireman licensed): man-days calculated from number of new points from Screen 4. Norm: 4 new points per day. → TOTAL MAN-DAYS — auto-summed. Used for duration calculation in Screen 17 and labour cost in Screen 18. |
| --- | --- |

Supervisor override: supervisor can adjust any calculated man-day figure if site conditions justify it. An override requires a note explaining the reason. Coordinator reviews all overrides before estimate is generated. Auto-calculated figures use industry norms from the platform's Zone Cost Database.

| 15 | Material Estimation Form 4 — auto-calculated quantities + brand tier selection → All quantities auto-calculated from Screen 13 measurements. Supervisor selects brand tier per material. → Floor tiles: quantity = (floor area + 10% waste) ÷ tile size. Shown in pieces and boxes. Brand tier: Premium / Value / Budget with example brands shown. → Wall tiles: quantity = net wall tile area + 10% waste. Shown in pieces and boxes. → Skirting tiles: quantity = skirting perimeter (auto from measurements). → Floor tile adhesive (bags 20kg): quantity = tile area ÷ 5 sq ft per kg coverage. Brand tier: Premium (Laticrete 307) / Value (Roff New Construction) / Budget (Weber Set-Plus). → Wall tile adhesive (bags 20kg): quantity = wall tile area ÷ 4 sq ft per kg. For large format or wet zone: high-flex adhesive (Laticrete 290 High Flex) mandatory — system auto-selects. → Waterproofing compound (kg): quantity = waterproofing area × 2 coats × 0.75 kg per sq ft per coat. Brand: Premium (Dr. Fixit Fastflex) / Value (Fosroc Brushbond) / Budget (Sika TopSeal 107). → Epoxy grout (kg): quantity = total grout area × 0.4 kg per sq ft for 2mm joints. Brands: Laticrete Stainfree / Roff Epoxy / Myk Arment. → Cement (bags 50kg): quantity = (plastering area + screed area) ÷ 35 sq ft per bag. → Coarse sand (CFT): cement bags × 3 CFT per bag. → Tile spacers and levelling clips: 1 full pack per 100 sq ft of tiling. → Plumbing materials: CPVC or uPVC pipe and fittings — lump sum per relocation from scope in Screen 3. → TOTAL MATERIAL COST — unit rates from Zone Cost Database (Noida NCR zone). Shown as estimated total. Breakdown by material item shown. → Three-tier comparison shown: Premium tier total / Value tier total / Budget tier total — supervisor notes client's preference and coordinator confirms. |
| --- | --- |

| 16 | Risk Flag Summary Auto-generated from all risk flags raised across Screens 2–12 → All risk flags raised during the visit are shown in one list. Each flag shows: the screen it was raised on, the description, the standard cost allowance from the Zone Cost Database, and whether the client can waive it (Yes/No). → Example flags shown on this screen: → ⚠ Screen 5 — Existing waterproofing failure at floor-wall junction. Recommended: additional crystalline layer. Standard allowance: ₹2,800. Waivable: Yes (client signs waiver, platform not liable for recurrence). → ⚠ Screen 3 — Aged copper supply pipe (estimated 25+ years). Recommended: full replacement. Standard allowance: ₹3,500. Waivable: Yes. → ⚠ Screen 4 — Non-IP rated light fitting in wet zone. Required: replacement to IP44. Standard allowance: ₹1,200. Waivable: No — this is a safety compliance requirement, not optional. → ⚠ Screen 2 — Structural crack on north wall. Required: civil engineer assessment before work begins. Waivable: No. → Total risk allowance shown: sum of all non-waived flags = ₹X added to estimate. → Supervisor adds any final notes on each flag. → No photographs required on this screen — all photographs already uploaded against the relevant checklist screen. |
| --- | --- |

| 17 | Timeline — Day-by-Day Work Schedule Form 3 — auto-generated from labour sheet + curing norms → Timeline auto-generated from: total man-days per trade (Screen 14) + mandatory curing wait times (from industry norms database) + working hours restriction (Screen 12). → Curing norms built into the auto-schedule (these are fixed, not adjustable by supervisor): → After plastering and screeding: minimum 24-hour surface drying before waterproofing can be applied. → Between 2 coats of waterproofing: minimum 12-hour dry time. → After waterproofing: minimum 24-hour water ponding test before tiling can begin. → After tile adhesive set: minimum 24-hour cure before epoxy grouting. → Wall tile vertical stacking: maximum 4 feet (2 rows) per day — adhesive must set before next rows are stacked. → Hydraulic pressure test on plumbing: 3–4 hours hold before wall chases are plastered closed. → Output shown as a day-by-day schedule: Day 1 — Demolition begins. Day 2 — Demolition complete + rubble cleared. Day 3 — Plumbing alterations begin. Day 7 — Waterproofing + ponding test. Day 8 — Tiling begins. And so on. → Total project duration shown in working days. Start date pulled from booking. End date calculated and shown. → If society noise restrictions (Screen 12) reduce working hours: duration automatically extended and total man-day cost adjusted. → Supervisor can flag if any timeline item is unrealistic given site-specific conditions — coordinator reviews before estimate is issued. |
| --- | --- |

| 18 | Payment Milestone Schedule Form 5 — auto-calculated from total labour cost → Total labour cost calculated from: man-days per trade (Screen 14) × zone day rates from Zone Cost Database. → Six milestone payment schedule auto-generated (based on Form 5 structure from uploaded document): → Milestone 1 — Mobilisation advance: 15% of total labour. Trigger: tools, machines, and crew on site and photographed on platform app. → Milestone 2 — Demolition clearance: 20%. Trigger: all old tiles and mortar chipped and hauled to ground level. Photograph evidence uploaded by supervisor. → Milestone 3 — Plumbing completion: 20%. Trigger: all pipe relocations complete, pressure-tested (3–4 hours hold documented), and wall chases sealed. Photograph and pressure test notation on app. → Milestone 4 — Waterproofing and base: 15%. Trigger: 2-coat waterproofing dry + 24-hour ponding test passed + floor screed slope set. Ponding test photograph mandatory. → Milestone 5 — Tiling completion: 20%. Trigger: all floor and wall tiles laid, levelling clips removed, skirting installed. Photograph of completed tiling. → Milestone 6 — Final retainage: 10%. Trigger: epoxy grouting complete + all fixtures refitted + 24-hour flood test passed + client snag-free sign-off on platform app. → Platform rule: no milestone payment is released without the supervisor uploading photograph evidence of the trigger condition and coordinator approving the release. Client receives a notification when each milestone is triggered. → Total labour cost and per-milestone amount shown. Material cost shown separately. |
| --- | --- |

| 19 | Submit Scope Report Final submission — sends report to coordinator and client → Progress bar shown: all 18 screens green — READY TO SUBMIT → Summary shown before submission: client name, job address, total floor area (verified), total wall area, number of risk flags, total man-days, estimated labour cost range, estimated material cost range, total project duration in working days, proposed start date, proposed end date. → Supervisor makes a final declaration: 'I confirm that all measurements are taken on site, all checklist items are based on direct observation, and all photographs uploaded are taken at this site visit on today's date.' → Aadhaar OTP confirmation — supervisor signs off digitally. → SUBMIT button — tap to submit. → On submission: scope report status changes to SCOPE REPORT SUBMITTED. Platform coordinator notified immediately. Coordinator review clock starts — target 2 hours to review and approve for estimate generation. Client notified: 'Your scope report is ready. Estimate will be sent within 24 hours.' → If supervisor tries to submit with any screen incomplete: submit button remains disabled. App shows list of incomplete screens with tap-to-go shortcut. → After submission: supervisor cannot edit the report. Any corrections go through coordinator with a note explaining the change. |
| --- | --- |

| 🔒 | SUBMIT BUTTON IS DISABLED until: all 18 screens complete AND all mandatory photographs uploaded AND Aadhaar OTP signed. The app shows a running count: 'X of 18 screens complete. Y photographs still needed.' This cannot be bypassed. |
| --- | --- |

Coordinator Review — 6 Gates Before Estimate is Triggered

After supervisor submits on Screen 19, the coordinator reviews before the estimate engine runs. Target review time: 2 hours. These 6 gates must all pass before the estimate is auto-generated.

| Gate | Review point | Pass criteria | Fail action |
| --- | --- | --- | --- |
| Gate 1 | All 18 screens complete | Progress bar shows 18/18 green | System prevents submission — cannot fail at this stage if submission was allowed |
| Gate 2 | Photographs count and quality | Minimum photographs per screen uploaded. Photographs are clear and show what they claim to show — not blurred, not of wrong subject. | Return to supervisor with specific screens to re-photograph. Supervisor must return to site or upload additional photographs if taken on site. |
| Gate 3 | Measurements verified | Measured floor area, wall area, and ceiling height are entered as numbers — not descriptions. Cross-check: if floor area × ceiling height is inconsistent with wall area, flag for review. | Request supervisor to re-measure and resubmit. Note the discrepancy. |
| Gate 4 | Risk flags reviewed | Each risk flag checked against the Zone Cost Database risk allowance library. Standard allowances confirmed. Non-standard or unusually high flags escalated to platform manager. | Override if non-standard. Note reason. Do not change supervisor's observed flag — only the cost allowance. |
| Gate 5 | Labour man-days plausibility | Auto-calculated man-days cross-checked against coordinator's experience with similar scope in this zone. Any figure more than 30% above the zone norm is flagged. | Request supervisor to confirm or adjust with a note. Coordinator can override after confirmation. |
| Gate 6 | Timeline feasibility | Auto-generated timeline reviewed for feasibility given start date, curing norms, and society noise hours. Any timeline that compresses a mandatory curing wait is flagged. | Adjust timeline. Notify supervisor and client of revised end date before estimate is issued. |

After all 6 gates pass: coordinator taps 'Approve for estimate.' The Nest estimate engine runs automatically — generating the elemental cost estimate from the verified scope × zone rate card. Estimate is sent to client within 30 minutes of coordinator approval.

Database Fields — scope_reports Table

Every field captured across Screens 2–18 maps to a column in the scope_reports table in Supabase. This table is linked to the jobs table via job_id. Below are the key field groups that must be included in the updated schema.

| Field group | Fields | Data type |
| --- | --- | --- |
| Identity | job_id, supervisor_id, visit_date, visit_start_time, visit_end_time, submission_timestamp | UUID, datetime |
| Measurements | floor_length_ft, floor_width_ft, floor_area_sqft, ceiling_height_ft, wall_n_area, wall_s_area, wall_e_area, wall_w_area, total_wall_area, net_tile_area | Decimal |
| Demolition | floor_slab_status, wall_substrate_condition, existing_tile_height_ft, target_tile_height_ft, load_bearing_wall_flag, structural_crack_flag | Enum, Boolean |
| Plumbing | wc_relocation_required, wc_shift_distance_ft, core_cutting_required, shower_relocation_required, shower_shift_ft, pipe_material, pipe_age_band, drain_condition | Enum, Boolean, Decimal |
| Electrical | exhaust_fan_status, wet_zone_ip_compliant, elcb_present, geyser_status, geyser_capacity_litres, new_points_count | Enum, Boolean, Integer |
| Waterproofing | damp_evidence_found, damp_location, efflorescence_found, tile_hollow_found, waterproofing_system_proposed, waterproofing_area_sqft | Boolean, Enum, Decimal |
| Wall condition | wall_n_condition, wall_s_condition, wall_e_condition, wall_w_condition, replastering_area_sqft, plastering_spec | Enum, Decimal |
| Floor condition | floor_condition, floor_fall_adequate, drain_type, screed_required, screed_spec | Enum, Boolean |
| Tile specification | floor_tile_format, floor_tile_type, floor_tile_brand_tier, wall_tile_format, wall_tile_type, wall_tile_brand_tier, grout_type, client_supplies_tiles | Enum, Boolean |
| Fixtures | ewc_action, basin_action, shower_action, geyser_action, accessories_count | Enum, Integer |
| Logistics | building_floor, lift_access, floor_height_rise_mm, door_trim_required, rubble_disposal_point, noise_hours_restriction | Enum, Boolean, Integer |
| Labour (auto-calculated) | demolition_mandays, debris_mandays, plumber_mandays, plastering_mandays, screed_mandays, tiling_mandays, waterproofing_mandays, electrical_mandays, total_mandays | Decimal |
| Risk flags | risk_flag_ids (array of risk_flag records), total_risk_allowance | UUID array, Decimal |
| Timeline | project_start_date, project_end_date, total_working_days, timeline_json (day-by-day schedule) | Date, Integer, JSON |
| Payment milestones | milestone_1_amount through milestone_6_amount, total_labour_cost | Decimal |
| Status | report_status (DRAFT / SUBMITTED / COORDINATOR_REVIEW / APPROVED / ESTIMATE_GENERATED), coordinator_id, approval_timestamp | Enum, UUID, datetime |
| Photographs | photo_urls (array of URLs with screen tag and checklist item tag) | JSON array |